MY AVIONICS SUBMISSION
ROUND 2

TASK 1:

I picked the Build 2 (Depth camera) for this mission.
Reason:
The mission demands good sensors to find odysseus, so prioritizing depth perception, AI capability is important and at the same time you have also given me the constraint for low weight ,to fly longer(12 mins) and, power consumption must be on the lower side.

The OAK-D LITE + AI has 3 cameras(1 4krgb and 2 greyscale for depth) it can measure a depth upto 8m of distance. It has AI acceleration and hence can provide direct human detection and depth data which has been preproccessed to the Rasberrypie 5.
The Rasberrypi 5 is a good general purpose and cheap board although I later found out Jetson Orin Nano is better at heavy duty AI work which honestly isn’t required since all we need to do is identify a dude. The Jason Orin Nano is slightly heavy(100-200g compared to 46g of pi) and draws more power. [There is also cost difference of more than 300$]

Here is priority of all parameters: from highest on top to lowest on bottom.

- Use case from statement (we are making the drone for a reason.)

- Computation (if you have a drone that reaches no destination due to inability of identifying its target, the mission is definitely pointless)

- Power consumption (for sustainability of 12mins)

- Weight

- Cost.

Heres why I rejected the other builds:

Build1 has IMX219-83 stereo sensor and also built in IMU(accelerometer , gyro etc),it has 3m depth perception due to its 60mm baseline. It lacks the AI computing power that 2 offers (my prioritization to computation) however I believe that part is balanced by the Jetson Orin Nano. This was honestly pretty confusing but I rejected this mainly due to the heavy power consumption and increased weight and high cost of Jetson.

Build 3 uses LiDAR. LIDAR is light detection and ranging. It is capable of 360 degree measurements however cannot detect people and hence the Arducam IMX219 wide angle camera is given(though that cant measure distance due to not being stereo). Reason for rejection is the usecase priority being we are taking the drone over ocean and lidar generally doesn’t do well over water since its I.R rays are scattered off or absorbed in water. The Arducam IMX219 WIDE again doesn’t have the extra AI computing power we need for detection of person.(I know pi does it but we comparing build 2). The wide angle also needs to be adjusted for distortions.

Build 4  &5 is again inefficient due to the IMX219 not having AI accelerator. The heavy and expensive Jetson Orin Nano also plays a role in my choice of 2(though it is pretty close to a good choice from what I’ve researched)

BONUS:

Orientation of cameras: We are going over sea to rescue Odysseus. We would be required to spot him if he was dying in the middle of the ocean where debris would have surrounded him (Poseidon breaks his boat). We also would require the camera to avoid obstacles,waves etc. I would orient the OAK D-Lite slightly downward toward the ocean(approx. 20 degrees from horizontal), keep the cameras parallel to the horizon and frame, and fix the camera right in front of the drone(I think this obvious). This helps the camera observe the ocean better and also helps avoid obstacles along the way. Stereo cameras must me horizontal for maintaining balance.

Alternate configuration: I would choose build 5. Build 5 is extremely good due to Jetson Orin Nano having enormous AI computing power. It however draws more power and has more weight but this limitation as you see in the calculation of part 3 doesn’t make the biggest impact to stay within given constraints. D345i provides excellent stereo depth, has built in IMU and captures in colour (RGB). Paired with Jetson, it can identify not only Odysseus but also the situation he is in. The limitation of D435i not having an ai accelerator is balanced by the greater computing power of the Jetson.

TASK 2:

Communication architectures and their role in our case.

An RC transmitter and receiver has the role of handling the core manual flight control of the drone. Its role in an autonomous flight(combined with VTX) is to ensure safety of the drone. If the flight computer on board or the UART from pi to pixihawk mini starts to malfunction we need a manual restabilization or rescue path.

A Telemetry module provides the real-time monitoring between the drone and our laptop on ground. Important to give situational awareness of the drone to the ground user.

A VTX is that device handling realtime transmission of video data captured by drone to laptop monitor. Gives real time data for people on ground if drone is working fine(Also if odysseus is alive if the ai processor fails to detect that in whatever case).

MODELS I AM SELECTING:
RC transmitter: Radiomaster pocket RP2 V2 ELRS 2.4GHz.

Range: about 800m to 1km (now I know I could have used radiomasterpocket er8 with 1.1 to 2km range but it is heavier weighing 16.8g compared to 0.5g. its compatability is also pwm which is different from ppm accepted by Pixhawk so you would have to make a crsf connection anyway)
Connection:It does not use ppm or pwm protocols and uses CRSF pure digital protocol.

Connected to Pixhawk by a UART port and wired RX of pix to TX of RC and vis versa.

Powering: powered by Pixhawk itself from its VCC/5V peripheral port.

Telemetry module:

I chose Holybro Sik Telemetry Radio V3. Choosing the 100mw version which operates from 5V source. It uses a 3.3V UART connection and is designed for Pixhawk flightcontroller.

Range: It has a range of over 300m.
Connection: uses MAVLink communication protocol and is connected to Pixhawk mini by UART interface.(#UART is the physical hardware protocol used through wires,voltage levels etc to move bits between 2 hardware points)

MAVLink messages are sent directly through the radio and these can be run on ground laptop through softwares like mission planner.

On the Pixhawk it is connected through TELEM1 port which is a explicit UART telemetry port.

Powering: It is powered by 5v peripheral port on Pixhawk itself.(Pixhawk maybe directly connected to power module ofc)

VTX:

I chose DJI O4. Provides Live video link and uses digital video system and not analogue.

Range: Analogue system has signal strength that decreases with range. Hence not considered for our use case. Range is 10-15km

Connection: NOT connected to Pixhawk. It directly wirelessly should transmit its video data to us.

Powering: Due to its high voltage rating of (5-9v) it is connected directly to battery via voltage regulator.

NOTE: I HAVE ATTACHED CANVA DRAWING WITH THIS IN REPO.(I was not well for a long time so did not have time to learn kicad in 2 days.)

TASK 3: Calculation

Weight < 2.5kg

Time of flight > 12mins

Thrust to weight ratio at 50% throttle is enough to hover the drone.(i.e >1=)

I am doing the calculation based on these following constraints:

Calculation:

I start by calculating the weight and power of each component (to find battery)

I am using the ESC as Holybro BLHeli-S 20A since it is part of the holybro kit and is recommended and highly compatable with our holybro motor. The motor capability maxes at 17A so 20A is very sufficient. Ill do the calculations and see if this needs to be changed based on our constraints.

Battery recommended for Holybro S500 V2 Motor 2216-920KV-CCW is the LiPo 4S series. I have to choose from 4S 5000 mah , 4S 6000mah , 4S 7000mah or maybe more based on capacity and weight. Let battery mass be m and rating be Q. All the batteries offer 16.8V when charged.

P = VI for those components whose power ratings aren’t fixed. I am borrowing the ratings from individual respective websites.

Power and weight of electronics

| COMPONENT | Weight | Power drawn | Current rating |
| --- | --- | --- | --- |
| Sensor OAK-DLite fixed focus | 61g | 5W | 1A |
| Flight computer Rasberry PI 5 | 62g | 8W (current rating 0.8A on website) heavy and 4W idle. | 2.4A |
| Pixhawk 6C mini | 59.3g | 1.5W | 0.29A |
| DJI O4 air | 9g | 10.5W | 2A BEC |
| Holybro Sik Telemetry Radio 100mW | 23.5g | 100mW | 0.10A TX |
| RadioMasterPocket RP2 V2 ELRS 2.4GHz | 0.55g | 200mW | 0.04A |
| Holybro BLHeli-S 20A | 4-8g | ESC and motor consume same current. Their joint power is written in motor section. | 20A |
| Extra Wiring + Frame | ~782g( ive considerd 800g) | 500mW | ~ |
| Holybro S500 V2 Motor 2216-920KV-CCW | 63g | 220W at peak load(accounting for mild ocean storm)(with ESC). | Calculated below. |
| TOTAL: | 1086.35g | 245.8 |  |

Thrust Calculation:

This is the Thrust for Holybro S500 V2 Motor 2216-920KV-CCW motor.

| Throttle | Voltage | Current/motor | Thrust/motor |
| --- | --- | --- | --- |
| 30% | 16.1 V | 1.46 A | 215 g |
| 50% | 16.08 V | 3.82 A | 470 g |
| 100% | 15.92 V | 17.42 A | 1367 g |

Constraint: it should hover at 50% throttle.

Thrust = 4xper motor

=4(470)

=1880

Thrust per weight at 50% throttle = 1880/1086.35 (note both are in g)=1.73ish

This is close to 1 so no problem.

ENERGY CALCULATION AND ENDURENCE:

Total Energy stored(E) =  V X Q

= 16.8(charged) x Q(charge in Ah).

Calculation for under 12 minutes( sustainability):

Energy by battery = E in Wh

Time = 12mins = 0.2h

Theoretical available power(nominal conditions) = E/0.2

= 16.8xQ/0.2 = 84Q

This should be less than the total power consumed by components ofcourse

P > 225.8W

84Q>225.8W

Q > 2.68809Ah

Q > 2688mAh

Mass of battery:

M + 1086.35< 2500

M < 913.65

Hence from all conditions we finally have:

M < 913.65g

And			Battery rating Q > 2688mAh

BATTERY:

This is a big range.

I chose the 4S 5000mah battery or 4S 6000mah battery as both have weights of

450-500g and 550-600g respectively. Even 7000 is possible but it would overdo for this mission.

Recalculating thrust ratio with 4S 6000 mah 600g battery weight:

1880/1686.35 =1.11 (fine)

Energy E = current (in Ah) x battery voltage = 6x16.8=100.8Wh

Endurence time = E/power =100.8Wh/245.8 = 0.41hours =24mins (since we use only  80% of battery ig you can take 19mins)

Total current:

Power from electronics:

From raspberry pi = vi=5(2.4)=12w

Oakdlite = 5x1 = 5w

DJI O4 air = 5(2) = 10w

Pixhawkmini 6c = 1.5w

Sik Telemetry Radio V3 = 5x(100ma ITX) = 0.5w

Radio master = 0.2w

Total power = 29.2W

Ielectronics = 29.2/16.8 = 1.738A [16.8 comes from battery charged fully]

Current from motors+ESC:

P = VI = 3.82 x 4

=15.28 (This is less than ESC rating so we good)

Itotal = Imotor+ESC + Ielectronics

=15.28 + 1.738

=17.018A

BONUS:

To find alternate motors with more thrust for lower power.

Trust to power ratio of current: Holybro S500 V2 (at 100% throttle) = 1367g/248.6W=5.4987 [power mentioned in table above is nominal power consumption]

Alternate Motor: SunnySky V3508 KV580 which consumes a power of 215W(including its respective esc for 30A) and producing 1400g thrust. this ratio comes to be: 1400/215 =6.5116 (Note that it does require a propeller of 15x5.5) which is a bigger propeller required for its lower rpm.)

If you are willing to accept a battery change, we can use a 6S battery with motor T-Motor MN4006 KV380 which has a ratio of 1561g/199W = 7.84!! (this also requires a standard propeller size of 15x5). [ I know a 6S battery adds more weight but the weight difference is close to 85grams atmost so we can handle it] (wattage value with esc)

With current propeller I honestly didn’t find any motor too good. Hope this works ; )